# Tutorial

# Model Conceptualisation,

# Non-Stationarity

# and

# Pilot Points

GMDSI

January, 2024

Table of Contents

1. Introduction	1

1.1 General	1

1.2 This Tutorial	1

2. The Model	2

2.1 Running the Model	2

2.1.1 A Few Site Details	2

2.1.2 Some Model Details	2

2.1.3 Some Model Output Files	3

2.1.4 Exporting Model Outputs to a GIS	3

2.2 Some Site Details	5

2.2.1 History-Matching Dataset	5

2.2.2 Parameterisation	6

3. The Conceptual Model	8

3.1 General	8

3.2 Plotting Local Anisotropy Axes	9

3.3 Interpolating Anisotropy Specifications to the Model Grid	10

3.3.1 General	10

3.3.2 Running PLPROC a First Time	11

3.3.3 Inspecting Interpolation Outcomes	12

3.3.4 Running PLPROC a Second Time	13

4. Pilot Points Parameterisation	16

4.1 Concepts	16

4.2 The PLPROC Script	16

4.3 Viewing Kh and Kz	18

5. Building a PEST Dataset	20

5.1 General	20

5.2 Parameters	20

5.2.1 Template Files	20

5.2.2 Creating a Partial PEST Control File	20

5.3 Observations	20

5.3.1 Site Sample File	20

5.3.2 Interpolation to Observation Sites and Times	21

5.3.3 Adding Observations to the PEST Control File	23

5.4 The Model Batch File	24

5.5 Running PEST	25

6. Prior Parameter Uncertainty	26

6.1 General	26

6.2 Interpolation from Conceptual Points to Pilot Points	26

6.2.1 Concepts	26

6.2.2 The PLPROC Script	26

6.3 Covariance Matrix Generation	27

6.4 Parameter Uncertainty File	27

6.5 Generating Random Parameter Samples	28

6.5.1 Concepts	28

6.5.2 Running RANDPAR	28

6.5.3 Running PEST	29

# 1. Introduction

## 1.1 General

This tutorial covers a lot of issues. Its primary purpose is to teach you how to work with non-stationary, anisotropic, parameter fields when using pilot points as a parameterisation device. However, in the process, you will learn some other things as well. These include:

- Some important aspects of PLPROC functionality;

- Use of PEST Groundwater Utility Suite programs to display model datasets in a GIS and SURFER;

- Automation of PEST setup in complex situations;

- Random parameter set generation.

Anisotropy is important in many modelling contexts. These contexts include alluvial systems, as well as hard rock areas that are pervaded by structure. Often the direction and magnitude of anisotropy can change from one place to another, with transitions that may be smooth or sharp. The propensity for anisotropy to vary spatially should be accommodated in site conceptualisation and in model parameterisation.

Where pilot points are used as a parameterisation device, anisotropy must be accommodated in two ways. The first is in the way that interpolation takes place from pilot points to a model grid. The second is in the formulation of prior parameter covariance matrices. Prior parameter covariance matrices serve two purposes. When undertaking model calibration, they are used for regularisation. When undertaking uncertainty analysis, they are used for stochastic parameter field generation. Failure to take account of parameter correlation, and the possibility that it may be anisotropic and spatially variable, will probably induce predictive bias.

## 1.2 This Tutorial

All files and programs that you need to undertake this tutorial are supplied. Before starting the tutorial, you should unzip the tutorial file, and then open a command-line window in the main tutorial folder. The documentation subfolder contains this document. The results folder contains a copy of all files that are produced as the tutorial unfolds. These files allow you to start the tutorial at a place that is good for you, and/or to recover from any mistakes that you may make along the way.

# 2. The Model

## 2.1 Running the Model

### 2.1.1 A Few Site Details

The model on which this tutorial focusses is loosely based on a real-world case.

The model domain is on the inside bend of a river. Part of the domain spans alluvial sediments that are close to the river. However, part of the domain spans material that is only thinly overlain by river alluvium, or not overlain by alluvium at all.

Figure 2.1. The model grid superimposed on alluvial subdivisions.

### 2.1.2 Some Model Details

The simulator is MODFLOW 6. Discretisation is of the DISV type. The model grid is quadtree-refined over part of its domain. We use MODFLOW 6 with an unstructured grid in order to illustrate the flexibility of the methodologies that are demonstrated herein.

The model has a single layer. However the tutorial discusses how workflow elements can be extended to multiple layers.

It is not the purpose of this tutorial to calibrate the model. The tutorial demonstrates model parameterisation and PEST setup only. A simple, steady-state, version of the model is provided. This does not simulate the river boundary condition. However, fixed heads are assigned to the northern and southern boundaries of the alluvial system using the MODFLOW 6 CHD package. This is all that is needed for water to flow, and for the model to run. Water flows from north to south.

To run the model, type the following command at the screen prompt.

> mf6

MODFLOW 6 looks for a name file named mfsim.nam. This cites the model-specific name file gwmod.nam. The NPF package input file is named gwmod.npf. Inspect this file. Its GRIDDATA block is reproduced below.

BEGIN GRIDDATA ICELLTYPE LAYERED IPRN 5 CONSTANT 0 # ICELLTYPE 1 K LAYERED IPRN 12 OPEN/CLOSE kh.dat END GRIDDATA

Figure 2.2. GRIDDATA block of file gwmod.npf.

It is apparent from the above figure that MODFLOW 6 obtains cell-based hydraulic conductivities from an external file named kh.dat.

Inspect gwmod.disv. It is apparent from this file that the model  grid is more extensive than shown in Figure 2.1. This figure shows only that part of the model grid in which cell IDOMAIN values are positive. IDOMAIN is set to -1 in inactive parts of the model domain. We will accommodate this when undertaking parameterisation and spatial interpolation.

### 2.1.3 Some Model Output Files

If asked to do so, MODFLOW 6 writes a binary file containing details of its grid. For the present model this file is named gwmod.disv.grb. Some of the programs of the PEST Groundwater Utility Suite read this file, together with model data that is resident in other files. Some of these programs produce files that can be used for two- and three-dimensional display of model inputs and outputs. We will use some of these programs during the present tutorial; other visualisation-enabling programs are discussed in other tutorials. However, the main focus of the present tutorial is PLPROC. PLPROC can also read a binary grid file that is written by MODFLOW 6.

In the present example, heads calculated by MODFLOW 6 reside in file gwmod.bhd. This is a binary file. You can learn about the contents of any binary file written by MODFLOW 6 (whether it is a system state or budget file) using the MF6ARRDET utility from the PEST Groundwater Utility Suite. Run it now from the command line, responding to its prompts as follows.

> mf6arrdet Program MF6ARRDET lists the contents of an unformatted MODFLOW6 output file. Enter name of binary MODFLOW6-generated file: gwmod.bhd Is this a dependent variable (eg. head) or cell-by-cell flow file? [d/c]: d Is the model grid structured (i.e. DIS), DISV or DISU? [s/v/u]: v Enter name for output file: temp.dat - 1 datasets read from file gwmod.bhd. - file temp.dat written ok.

If you view the contents of temp.dat you will see that gwmod.bhd contains only a single heads array, this corresponding to a simulation time of 1.0. If you are interested, you can also obtain a description of the arrays that reside in the other MODFLOW 6 binary output file gwmod.cbc. Its contents are a little more interesting, but are not used in this tutorial.

### 2.1.4 Exporting Model Outputs to a GIS

We will first extract the single array of model-calculated heads from gwmod.bhd. We will then record this array in a GIS-readable file that contains grid and IDOMAIN details together with these heads.

First run the MF6BIN2TAB_H utility. (This is a member of the PEST Groundwater Utility Suite.)

> mf6bin2tab_h Program MF6BIN2TAB_H tabulates nodal system states pertaining to a user-nominated simulation time as read from a MODFLOW6 binary dependent variable file. Enter name of binary MF6-generated file: gwmod.bhd Is the model grid structured (i.e. DIS), DISV or DISU? [s/v/u]: v How many layers in the model? 1 Enter simulation time (i.e. TOTIM) of interest: 1.0 Enter name for node data table file: heads.ndt - file heads.ndt written ok.

Edit file heads.ndt. (“ndt” stands for “node data table”.) Alter “NODE_NUMBER” (the header on the first column) to “icpl”. (This specifies the cell index in a single layer of a MODFLOW 6 DISV grid.) Then run MF62GIS (another member of the Groundwater Utility Suite), responding to its prompts as follows.

> mf62gis Program MF62GIS writes a BLN file and MIF/MID files for a MODFLOW6 model. Enter name of MODFLOW6 binary grid file: gwmod.disv.grb - file gwmod.disv.grb read ok. Enter layer number of interest: 1 Enter name for BLN file (<Enter> if none): <Enter> Enter filename base for MIF/MID files (<Enter> if none): grid Enter name of tabular data file (<Enter> if none): heads.ndt The following columns have been detected in the table file. Indicate whether you would like pertinent data transferred to MIF/MID file. Data in column labelled "LAYER"? [y/n]: n Data in column labelled "CELL"? [y/n]: n Data in column labelled "HEAD"? [y/n]: y Does column contain integer or real data? [i/r]: r Enter value for missing data: 0 Add or replace values for duplicated cells [a/r]: r - reading tabular data file... - 8900 lines of data read from file heads.ndt. - file grid.mif written ok. - file grid.mid written ok.

The MIF/MID files (“MIF” stands for “MapInfo Interchange Format”) that are written by MF62GIS can be read by any GIS and by SURFER. You can use these platforms to plot the grid, together with heads pertaining to grid cells, after filtering out cells for which IDOMAIN is not equal to 1. The following picture was plotted using QGIS. (Note that head values are not important in this tutorial. Our main concern is with parameterisation and PEST setup. As stated above, we will not be calibrating this model.)

Figure 2.3. Model grid with cells shaded according to head. Water flows from north to south.

## 2.2 Some More Site Details

### 2.2.1 History-Matching Dataset

Figure 2.4 shows the locations of wells at which we will pretend, for the purpose of this tutorial, that heads have been measured. (In the real-world site on which this case is loosely based, heads in these wells respond to changes in river elevations, and to the switching on and off of local pumps.) Observation wells have a higher spatial density in the region of quadtree grid refinement. This is why the grid is quadtree-refined in this area. The locations of these wells are recorded in a file named calibration_wells.dat. We will use this file later when constructing a PEST dataset.

Figure 2.4. Locations of observation wells.

### 2.2.2 Parameterisation

The model is parameterised using pilot points. The locations of these points are shown in Figure 2.5 together with observation wells.

Figure 2.5. Pilot points are shown in red. Observation wells are shown in blue.

The locations of these pilot points are provided in a file named ppmult.dat. This file will be read by PLPROC. It can also be used for plotting purposes. We will explain its contents shortly, together with the way in which pilot points are used to parameterise the model in a way that is in accordance with current site concepts.

# 3. The Conceptual Model

## 3.1 General

As is often stated, numerical modelling should be based on a sound conceptual model. This conceptual model should express that which is known about a site. At the same time, it should acknowledge that which is unknown about a site. This allows it to provide a basis for formulation of a prior parameter probability distribution – an essential ingredient of decision-support modelling.

In the present synthetic example, the conceptual model is encapsulated in numbers that are assigned to a set of points that are spread throughout the model domain. There are 30 of these points in all. We refer to them as “conceptual points” in order to distinguish them from pilot points that will be used for model parameterisation. In formulating the conceptual model in this way, it is ensured that there are enough conceptual points to enable interpolation (rather than extrapolation) from them to the model domain.

The following numbers are assigned to each conceptual point. We will refer to these numbers as “hyperparameters”, as most of them pertain to the prior probability distribution of model parameters, rather than to the parameters themselves.

- broadscale horizontal hydraulic conductivity (i.e. the local mean value of horizontal hydraulic conductivity);

- the correlation length of local spatial heterogeneity of horizontal hydraulic conductivity (i.e. Kh) about these broadscale values (we use the “a” value of an exponential correlation decay function to express this correlation length – see below);

- the anisotropy ratio pertaining to local heterogeneity;

- the bearing of the principle axis of local anisotropy.

Ideally, these numbers will have emerged from site characterisation studies. They are listed in a file named conceptual.dat that is supplied with this tutorial. The locations of these conceptualisation points are depicted in Figure 3.1.

Figure 3.1. Conceptualisation points.

Spatial correlation is expressed by the following decay function:

Ch=C0e-h/a							(3.1)

The correlation length assigned to each conceptual point pertains to the major axis of the local ellipse of anisotropy. The direction of this axis is described by its bearing (with respect to north). The anisotropy ratio is the ratio of the length of the major axis of the correlation ellipse to that of its minor axis. These axes are orthogonal. The anisotropy ratio is generally greater than 1.0.

We assume that for both Kh and Kz (see below), C0 of equation (3.1) is 0.0625. This is the variance of spatial variability. It is applied to the log (to base 10) of Kh and Kz. Variance is the square of standard deviation. The standard deviation of spatial variability is therefore 0.25, or a quarter of an order of magnitude.

## 3.2 Plotting Local Anisotropy Axes

The HANIS2BLN utility (a member of the PEST Groundwater Utility Suite) can write a BLN file that enables plotting of anisotropy axes at conceptual points. Run HANIS2BLN, responding to its prompts as follows.

> hanis2bln Program HANIS2BLN reads a columnar data file in which values for correlation length, anisotropy bearing and anisotropy ratio are provided at a set of points. It writes a BLN file that allows display of these. Enter name of anisotropy specification file: conceptual.dat Enter column number to read point x coordinate: 2 Enter column number to read point y coordinate: 3 Enter column number to read correlation length: 5 Enter column number to read anisotropy ratio: 6 Enter column number to read anisotropy bearing: 7 Skip a line at top of file? (y/n): y Enter name for BLN output file: conceptual.bln - file conceptual.dat read ok - file conceptual.bln written ok.

If conceptual.bln is imported into SURFER (or any other program that can read a BLN file), the following picture can be plotted.

Figure 3.2. Major and minor axes of anisotropy.

It is apparent that both the correlation length and the anisotropy ratio are greatest near the river. The direction of the major axis of anisotropy follows the course of the river. In contrast, the anisotropy ratio is 1.0 along the eastern margin of the model domain.

Note that where the anisotropy ratio is 1.0, the direction of anisotropy does not matter as far as assignment of hydraulic properties to the model grid is concerned. However, in the present case, we will shortly interpolate anisotropy bearing from conceptualisation points to model grid cells. Hence it makes sense to assign anisotropy bearing values to the eastern conceptual points (where the anisotropy ratio is 1.0) in a way that enables sensible interpolation between them and neighbouring conceptual points that overly alluvium.

Note also that anisotropy bearings should be expressed in a way that prevents discontinuous interpolation. Hence bearings to the west of north should be expressed as negative numbers rather than as high numbers that approach 360 degrees.

## 3.3 Interpolating Anisotropy Specifications to the Model Grid

### 3.3.1 General

Our next task is to interpolate anisotropy ratio and anisotropy bearing to every active cell in the model grid. These will allow us to conduct anisotropic interpolation from conceptualisation points, and from pilot points, to the model grid.

There is a problem, however.

Ideally, interpolation of anisotropy angle and bearing should account for the pervasive anisotropy (and spatial variation thereof) that these numbers proclaim. In order to accomplish this, interpolation of these quantities will be undertaken twice. Once this has been done, the assignment of anisotropy ratio and bearing values to all model grid cells can provide a framework for further interpolation of any other quantity to the model grid.

### 3.3.2 Running PLPROC a First Time

Interpolation will be undertaken using PLPROC. A PLPROC input file named plproc1.dat has been prepared for you. Some details of the script that is contained in this file are now provided. (See also comments in the PLPROC script file itself.) In this, and all other descriptions of PLPROC scripting, a script fragment is presented first; this is followed by an explanation of what the fragment does.

cl_mf6 = read_mf6_grid_specs(file=gwmod.disv.grb, & dimensions=3, & slist_idomain=idomain_mf6)

PLPROC is instructed to read the binary grid file written by MODFLOW 6. Among other things, this file informs PLPROC of the coordinates of all model grid cell centres; these are the locations to which spatial interpolation must take place. A CLIST named cl_mf6 is created, along with an SLIST named idomain_mf6. The latter SLIST hosts model cell IDOMAIN values. By the way, a CLIST is a list of coordinates. These coordinates can host integers (in SLISTs) and real numbers (in PLISTs).

cl_ppc = read_list_file(skiplines=1, & dimensions=2, & plist=kh_c_ppc;column=4, & plist=a_ppc;column=5, & plist=hanis_ratio_ppc;column=6, & plist=hanis_bearing_ppc;column=7,& id_type='character', & file='conceptual.dat')

The file which holds conceptual points and conceptual details (i.e. conceptual.dat) is read. A new CLIST named cl_ppc is created based on these points. All columns of conceptual.dat are read. Data pertaining to each column is associated with a separate PLIST.

calc_kriging_factors_auto_2d(target_clist=cl_mf6;select=(idomain_mf6==1), & source_clist=cl_ppc, & file=factors_conceptual.dat)

In preparation for spatial interpolation, kriging factors are calculated from the conceptual CLIST to the model grid CLIST – but only for model grid cells for which the IDOMAIN value is 1. The PLPROC function which is used to calculate these factors bases kriging on a spatially-variable variogram that it calculates itself based on distances between points from which interpolation must take place. Kriging factors are stored in a file named factors_conceptual.dat.

hanis_ratio_mf6=new_plist(reference_clist=cl_mf6,value=1.0) hanis_bearing_mf6=new_plist(reference_clist=cl_mf6,value=0.0)

Two grid-based PLISTs are defined to house the outcomes of the upcoming spatial interpolation process.

hanis_ratio_mf6=hanis_ratio_ppc.krige_using_file(file=factors_conceptual.dat, & transform=none) hanis_bearing_mf6=hanis_bearing_ppc.krige_using_file(file=factors_conceptual.dat, & transform=none)

In the above script fragment, the krige_using_file() function employs previously-calculated kriging factors to interpolate from conceptual points to model grid cells whose IDOMAIN values are 1.

write_column_data_file(header='yes', & file='concept_data.ndt';delim=space, & clist_spec='id';headtext='icpl', & slist=idomain_mf6, & plist=hanis_ratio_mf6, & plist=hanis_bearing_mf6)

This last of these script fragments has nothing to do with spatial interpolation at all. It instructs PLPROC to write a file named concept_data.ndt in which the grid-based outcomes of spatial interpolation are recorded in columns. With a little further processing (see below), this will make it easy for us to view the outcomes of the spatial interpolation process. We can then judge whether these outcomes are satisfactory.

To accomplish all of the outcomes described above, run PLPROC using the following command.

> plproc plproc1.dat

### 3.3.3 Inspecting Interpolation Outcomes

The MF62GIS utility (supplied with the PEST Groundwater Utility Suite) can be used to create a MIF/MID file pair based on the outcomes of the above interpolation process. Run MF62GIS as follows.

Program MF62GIS writes a BLN file and MIF/MID files for a MODFLOW6 model. Enter name of MODFLOW6 binary grid file: gwmod.disv.grb - file gwmod.disv.grb read ok. Enter layer number of interest: 1 Enter name for BLN file (<Enter> if none): <Enter> Enter filename base for MIF/MID files (<Enter> if none): conceptual1 Enter name of tabular data file (<Enter> if none): concept_data.ndt The following columns have been detected in the table file. Indicate whether you would like pertinent data transferred to MIF/MID file. Data in column labelled "idomain_mf6"? [y/n]: y Does column contain integer or real data? [i/r]: i Enter value for missing data: 0 Add or replace values for duplicated cells [a/r]: r Data in column labelled "hanis_ratio_mf6"? [y/n]: y Does column contain integer or real data? [i/r]: r Enter value for missing data: 1 Add or replace values for duplicated cells [a/r]: r Data in column labelled "hanis_bearing_mf6"? [y/n]: y Does column contain integer or real data? [i/r]: r Enter value for missing data: 0 Add or replace values for duplicated cells [a/r]: r - reading tabular data file... - 8900 lines of data read from file concept_data.ndt. - file conceptual1.mif written ok. - file conceptual1.mid written ok.

The following picture shows anisotropy ratio interpolated to the model grid. In case you are interested, I prepared this picture by first importing conceptual1.mif and conceptual1.mid into QGIS. I then removed cells in which idomain_mf6 was not equal to 1. I saved the result as a shapefile and imported this shapefile into SURFER for colouring according to hanis_ratio value. (I like SURFER’s colour options more than those of QGIS.)

Figure 3.3. Outcomes of spatial interpolation coloured according to hanis_ratio. The colour range is from 1.0 to 2.5.

The results are good, but not great. See, in particular, the south-west margin of the model domain where high values of hanis_ratio tend to cling to the conceptualisation points. This is an outcome of non-anisotropic interpolation.

To rectify this problem, we will now interpolate again. This time, the spatial interpolation process will be governed by grid-based values of anisotropy ratio and bearing that have just been calculated.

### 3.3.4 Running PLPROC a Second Time

See file plproc2.dat. This is a continuation of file plproc1.dat with postponement of the production of concept_data.ndt until after a second stage of interpolation.

calc_kriging_factors_auto_2d(target_clist=cl_mf6;select=(idomain_mf6==1), & source_clist=cl_ppc, & anis_ratio_plist=hanis_ratio_mf6, & anis_bearing_plist=hanis_bearing_mf6, & enforce_strong_anis=yes, & file=factors_conceptual_hanis.dat)

The above script fragment calculates another set of kriging factors; once again, this is accomplished using function calc_kriging_factors_auto_2d(). However, on this occasion the function is called with extra arguments. These tell it to use previously-calculated grid-based anisotropy ratio and bearing to assist with the interpolation. Internally, distances that are used to implement kriging undergo local adjustment in order to accommodate local anisotropy. New interpolation factors that are calculated using this function call are stored in a file named factors_conceptual_hanis.dat.

hanis_ratio_mf6=hanis_ratio_ppc.krige_using_file(file=factors_conceptual_hanis.dat, & transform=none) hanis_bearing_mf6=hanis_bearing_ppc.krige_using_file(file=factors_conceptual_hanis.dat, & transform=none)

In the above script fragment, anisotropy ratio and bearing are re-interpolated from conceptual points to the MODFLOW 6 grid using these new factors. These same factors can now be used to spatially interpolate the other two items of site conceptualisation from conceptualisation points to the model grid. These are correlation length and broadscale horizontal hydraulic conductivity.

kh_c_mf6=new_plist(reference_clist=cl_mf6,value=1.0) a_mf6=new_plist(reference_clist=cl_mf6,value=1.0)

First, new grid-based PLISTs are defined in the above script fragment using function new_plist().

kh_c_mf6=kh_c_ppc.krige_using_file(file=factors_conceptual_hanis.dat, & transform=log) a_mf6=a_ppc.krige_using_file(file=factors_conceptual_hanis.dat, & transform=log)

Then, function krige_using_file() uses the new interpolation factors to populate the model grid with values of broadscale (i.e. average) hydraulic conductivity and heterogeneity correlation length.

write_column_data_file(header='yes', & file='concept_data.ndt';delim=space, & clist_spec='id';headtext='icpl', & slist=idomain_mf6, & plist=kh_c_mf6, & plist=a_mf6, & plist=hanis_ratio_mf6, & plist=hanis_bearing_mf6)

The above call to function write_column_data_file() records the outcomes of all four conceptual-points-to-grid interpolation processes to a node data table file named concept_data.ndt.

Run PLPROC using the following command to implement all of the above function calls.

> plproc plproc2.dat

Program MF62GIS can now be used, once again, to write a MIF/MID file pair through which interpolation outcomes can be imported into a GIS and/or SURFER. MF62GIS keyboard responses have been prepared for you; they are stored in file mf62gis_conceptual.in. Run MF62GIS using the following command.

> mf62gis < mf62gis_conceptual.in

Plotting of anisotropy ratio achieved through use of these second-stage interpolation factors reveals a much better result. Compare Figure 3.4 with Figure 3.3.

Figure 3.4. Outcomes of interpolation carried out with level 2 interpolation factors coloured according to hanis_ratio. The colour range is from 1.0 to 2.5.

# 4. Pilot Points Parameterisation

## 4.1 Concepts

The locations of pilot points are shown in Figure 2.5. These were emplaced manually – with greater spatial density awarded to areas with greater data density.

Two parameters are associated with each pilot point.

The first are multipliers to apply to conceptual broadscale Kh. These multipliers can be adjusted during history-matching. They can also be used for stochastic expression of local heterogeneity. Recall that broadscale Kh values have been interpolated from conceptual points to the model grid with conceptual anisotropy (and spatial variation thereof) taken into account. The conceptual model also provides spatially-variable values of correlation length. These, too, have been spatially interpolated from conceptual points to the model grid. These will be applied to the Kh multipliers that express local heterogeneity.

Because the model possesses only a single layer, we do not need to worry about Kz. Nevertheless, Kz parameterisation will be introduced in order to illustrate ways of handling it that may be useful in multi-layer modelling contexts.

As part of our site conceptual model, let us presume that broadscale Kz can be calculated from broadscale Kh using a global factor. This factor is, in fact, calibration-adjustable. We further assume that local spatial variability of Kz can be represented using a set of local Kz multipliers. One of these is associated with each pilot point. We will assume that the geostatistics of these Kz multipliers are the same as those of the Kh multipliers. (That is, their variance, correlation length and anisotropy characteristics are the same.) However the Kz multipliers are statistically independent of the Kh multipliers.

Multipliers for both Kh and Kz are contained in the ppmult.dat pilot points file These are listed in the final two columns of this file. These columns have been populated with random values of these quantities. These random values are actually spatially correlated (in order to conform with the site conceptual model). The way in which correlated random values such as these are obtained will be explained shortly.

The PLPROC script will now be extended to accommodate this aspect of the model’s parameterisation. The MODFLOW 6 grid will be populated with grid-based values of Kh and Kz.

## 4.2 The PLPROC Script

See file plproc3.dat. This is a continuation of plproc2.dat. Its contents are now described. As usual, a fragment of the script is presented followed by a description of what that fragment does. See also comments in the PLPROC script file itself.

cl_ppm = read_list_file(skiplines=1, & dimensions=2, & plist=khmul_ppm;column=4, & plist=kzmul_ppm;column=5, & id_type='character', & file='ppmult.dat')

In the above script fragment, function read_list_file() is employed to read columns 4 and 5 of file ppmult.dat. These columns contain local multipliers for horizontal and vertical hydraulic conductivity. Each of these multipliers is associated with a pilot point. Internally to PLPROC, these multipliers are assigned to a new CLIST named cl_ppm that is ascribed to this set of pilot points.

Kriging factors must now be calculated from pilot points to the model grid.

calc_kriging_factors_sva_2d(target_clist=cl_mf6;select=(idomain_mf6==1), & source_clist=cl_ppm, & variogram=exponential, & kriging=ordinary, & a_plist=a_mf6, & anis_ratio_plist=hanis_ratio_mf6, & anis_bearing_plist=hanis_bearing_mf6, & min_points=1, & max_points=30, & search_radius=1e15, & file=factors_multipliers.dat)

Kriging factors are calculated using a new interpolation function named calc_kriging_factors_sva_2d(). This function acknowledges the fact (embodied in the site conceptual model) that kriging must be based on a spatially varying variogram. Following previous interpolation processes, hyperparameters associated with this spatially varying variogram have been ascribed to model grid cells. These are used to enable kriging based on spatially variable geostatistics. Kriging factors are stored in a file named factors_multipliers.dat. Notice that a selection equation is used to restrict calculation of kriging factors to that part of the model grid where IDOMAIN is equal to 1; this is the active part of the model grid.

Next we prepare for interpolation of these factors to the model grid.

khmul_mf6=new_plist(reference_clist=cl_mf6,value=1.0) kzmul_mf6=new_plist(reference_clist=cl_mf6,value=1.0)

In the above script fragment, two new grid-based PLISTs are defined. In a moment, these will host the outcomes of interpolation of Kh multipliers and Kz multipliers to the model grid.

khmul_mf6=khmul_ppm.krige_using_file(file=factors_multipliers.dat, & transform=log) kzmul_mf6=kzmul_ppm.krige_using_file(file=factors_multipliers.dat, & transform=log)

Notice that, in the above script fragment, transform is set to log. This is usual practice when interpolating quantities such as hydraulic conductivity (or their multipliers) which can vary over orders of magnitude, and whose variabilities are normally described using a log variogram.

In the next part of the script, the model grid population sequence is interrupted in order to read the global Kz multiplier from an external file. In PLPROC parlance this is known as a “scalar file”.

read_scalar_file(file=scalars.dat, valuecolumn=2, namecolumn=1)

File scalars.dat is reproduced below.

kzmul_global 0.1

Figure 4.1. File scalars.dat.

File scalars.dat contains a single number. The name of the variable that is associated with this number is kzmul_global. This variable will be used in future PLPROC processing. There is a reason that this number is read from an external file and not defined internally in the PLPROC script. It is because this number will shortly be denoted as a history-match-adjustable parameter. (By the way, a PLPROC scalar file can hold many such numbers.)

kh_mf6=kh_c_mf6*khmul_mf6 kz_mf6=kh_c_mf6*kzmul_mf6*kzmul_global

The above two lines of script complete population of the model grid with values of Kh and Kz. Two new PLISTs are defined in the process. Grid-based conceptual values of broadscale Kh are multiplied by numbers that have just been spatially interpolated from pilot points to the model grid; one of these sets of multipliers pertains to Kh while the other pertains to Kz. Kz values are further multiplied by the global Kz multiplier.

We would like to visualize the Kh and Kz hydraulic property fields that we have just created. So we record them in a node data table file.

write_column_data_file(header='yes', & file='kh_kz.ndt';delim=space, & clist_spec='id';headtext='icpl', & slist=idomain_mf6, & plist=kh_mf6, & plist=kz_mf6)

The above script fragment specifies that this node data table file is named kh_kz.ndt.

Finally, model input files are populated with values of Kh and Kz.

write_column_data_file(header='no', & file='kh.dat';delim=space, & plist=kh_mf6) write_column_data_file(header='no', & file='kz.dat';delim=space, & plist=kz_mf6)

The model input files are named kh.dat and kz.dat. Because the model has only a single layer, Kz is of no use (and isn’t even read by MODFLOW 6). As stated above, we are populating a model-based Kz array just to show how it can be done. The model does, however, use Kh. As is discussed above, the MODFLOW 6 NPF package directs the model’s attention to kh.dat to read cell-based values of Kh using an OPEN/CLOSE statement.

Using the above write_column_data_file() function calls, Kh and Kz values are recorded in their respective files in a single column. More sophisticated ways of writing model input files are provided by other PLPROC functions. See, for example, functions write_in_sequence() and replace_column().

Run PLPROC as follows.

> plproc plproc3.dat

## 4.3 Viewing Kh and Kz

Program MF62GIS can now be used to write MIF/MID files in which Kh and Kz values are recorded. These files can be imported into a GIS or SURFER wherein cells with negative IDOMAIN values can be removed and the contents of the remaining cells can be coloured.

MF62GIS keyboard responses have been prepared for you. You can run it using the following command. Alternatively, you can provide the keyboard responses yourself using file mf62gis_kh_kz.in as your guide.

> mf62gis < mf62gis_kh_kz.in

Maps of Kh and Kz are provided in Figure 4.2 and 4.3.

Figure 4.2. Map of Kh. Colouring is logarithmic. Kh values span the interval from 0.33 m/day to 18.6 m/day.

Figure 4.3. Map of Kz. Colouring is logarithmic. Kz values span the interval from 0.024 m/day to 1.92 m/day.

It is important to note that many variations of the above workflow are possible. For example, Kh and Kz values could have been clipped at user-specified lower/upper limits that are themselves spatially variable. The function through which broadscale Kz is calculated from broadscale Kh could have been nonlinear. PLPROC was designed to provide a high degree of flexibility.

# 5. Building a PEST Input Dataset

## 5.1 General

As has already been discussed, the above-described model will not be history-matched in this tutorial. The intention of this tutorial is to demonstrate a pre-history-matching workflow that facilitates translation of site concepts into model parameterisation. Once this has been done, a maximum a posteriori (i.e. MAP) parameter set can then be attained through model calibration using PEST_HP. Alternatively, or as well, the posterior parameter probability distribution can be sampled using PESTPP-IES.

Nevertheless, this section will demonstrate how easy it is to construct a PEST input dataset based on what has been accomplished so far.

## 5.2 Parameters

### 5.2.1 Template Files

Our model has three sets of parameters. These are:

- Kh multipliers at pilot points (there are 113 of these);

- Kz multipliers at pilot points (there are 113 of these);

- A global average Kh to global average Kz multiplier (there is just 1 of these).

The first step in creating a PEST input dataset is to make template files of model input files that house these parameters. These model input files are ppmult.dat and scalars.dat respectively. Template files have been created for you. They are named ppmult.tpl and scalars.tpl. Inspect these files. The parameter naming scheme is obvious.

If you wish, you can check the integrity of these files using the TEMPCHEK utility.

> tempchek ppmult.tpl > tempchek scalars.tpl

### 5.2.2 Creating a Partial PEST Control File

The TPL2PST utility can now be used to create and populate a “partial PEST control file” based on these parameters. TPL2PST is a utility program that is supplied with PEST. It requires a user-prepared input file to guide it in construction of the partial PEST control file. Such a file has been prepared; it is named tpl2pst.dat. If you inspect this file, you can see how it sets some of the specifications for the partial PEST control file that TPL2PST must write. See Part 2 of the PEST manual for further details.

Run TPL2PST using the following command to create a partial PEST control file named case.pst.

> tpl2pst tpl2pst.dat case.pst

Inspect file case.pst to see what TPL2PST has done. It has populated the “parameter groups”, “parameter data” and part of the “model input/output” sections of a new PEST control file. It has also written the “control data” and “singular value decomposition” sections of this file, providing these sections with sensible default values for control variables.

## 5.3 Observations

### 5.3.1 Site Sample File

The locations of observation wells are depicted in Figure 2.4.

As has already been explained, our simple model runs under steady state conditions. An inspection of the MODFLOW 6 time discretisation file gwmod.tdis reveals that the simulation time is 1 day. (The “simulation time” makes no difference to steady-state calculations. However temporal book-keeping in this manner enables seamless transition between use of steady state and transient observations in model history-matching.)

File observ.smp lists observed heads on 2nd January 2023. Note that we employ the “dd/mm/yyyy” protocol for representation of dates in this file. We could have employed the “mm/dd/yyyy” protocol if we wished. Our choice is recorded in a file named settings.fig which must reside in the current working folder. Many of the programs of the PEST Groundwater Utility Suite require the presence of this file so that they know which of these protocols to use for representation of dates.

File observ.smp is a so-called “site sample file”. This type of file is used by many of the programs of the PEST Groundwater Utility Suite. It can host both steady state and transient datasets. Transient datasets can ascribe more than one measurement to each observation site. Site sample file protocols are provided in Part A of the manual of the Groundwater Utility Suite. If you are unsure whether you have properly observed these protocols when building a site sample file, you can check its contents using the SMPCHEK utility. Run SMPCHEK by typing its name at the command line prompt.

> smpchek Program SMPCHEK checks the integrity of a bore sample file. Enter name of bore sample file: observ.smp - 39 lines read from bore sample file observ.smp - no errors found. - sample values found for 39 different bores.

### 5.3.2 Interpolation to Observation Sites and Times

Models like MODFLOW 6 calculate system states at the locations of active cell centres. These states must be spatially and temporally interpolated to the sites and times at which field measurements have been made if these measurements are to be used for model history-matching. There are a number of ways to do this. In the present case, we will not use the MODFLOW 6 OBS package, for an alternative approach is easier. (This is not always the case.)

MODFLOW 6 records an array of steady state heads in file gwmod.bhd. As has already been noted, these pertain to a simulation time of 1 day. These array-based heads will now be interpolated to the sites at which head observations were made. In doing this we will create a model-generated counterpart to file observ.smp. This new file will be named model.smp.

Spatial interpolation from a MODFLOW 6 unstructured grid to an arbitrary set of points is a two-step process. First interpolation factors are calculated. Then these are applied. Program MF6INTFAC (from the PEST Groundwater Utility Suite) can be used to calculate interpolation factors. MF6INTFAC can be used in conjunction with both DIS and DISV MODFLOW 6 models. For DISV models, cells can be of arbitrary shape.

Recall from Section 2 of this document that the locations of observation wells are recorded in file calibration_wells.dat. Inspect this file. It has four columns. The second and third columns contain well coordinates, while the last column contains well layer numbers.

Now run MF6INTFAC, responding to its prompts as follows.

> mf6intfac Program MF6INTFAC calculates interpolation factors from a MODFLOW 6 DIS or DISV grid to observation points. Enter name of MODFLOW6 binary grid file: gwmod.disv.grb - file gwmod.disv.grb read ok. Enter name of bore coordinates file: calibration_wells.dat - 39 bores and coordinates read from bore coordinates file calibration_wells.dat Enter name of bore listing file: calibration_wells.dat - 39 bores read from bore listing file calibration_wells.dat Enter name for interpolation factor output file: observ_factors.dat Enter name for enclosing polygon BLN file: observ_factors.bln - file observ_factors.dat written ok. - file observ_factors.bln written ok.

The BLN file that is recorded by MF6INTFAC identifies lines that join model grid cell centres from which interpolation takes place to observation points. In doing this, it creates polygons which surround these observation points. In the present case these polygons are rectangles (because the model grid is rectilinear). In other cases they are triangles.

The MF6MOD2OBS utility can now be run. It uses the above-calculated factors to interpolate model outputs to the sites of observation wells. It also undertakes time-interpolation from model output times to the times at which field measurements were made. (This, of course, is more important when history-matching a transient model than when history-matching a steady-state model.) Run MF6MOD2OBS, responding to its prompts as follows.

> mf6mod2obs Program UMF6MOD2OBS writes a bore sample file of a time-varying MODFLOW6-calculated dependent variable, interpolated to the sites and times supplied in an existing bore sample file. Enter name of node-to-bore interpolation file: observ_factors.dat - 39 bores and associated nodal interpolation factors read from node-to-bore interpolation file observ_factors.dat. Enter name of bore listing file: calibration_wells.dat - 39 bores read from bore listing file calibration_wells.dat Reapportion interpolation factors if any dry/inactive nodes [y/n]? y Enter name of existing bore sample file: observ.smp Enter name of binary MF6-generated file: gwmod.bhd Enter value signifying inactive cells [1.0E30]: 1e30 Enter value signifying dry cells [-1.0E30]: 1e30 Is the model grid structured (i.e. DIS), DISV or DISU? [s/v/u]: v How many layers in the model? 1 Enter time units used by model (yr/day/hr/min/sec) [y/d/h/m/s]: d Enter simulation starting date [dd/mm/yyyy]: 1/1/2023 Enter simulation starting time [hh:mm:ss]: 0:0:0 If a sample time does not lie between model output times, or if there is only one model output time, value at the sample time can equal that at nearest model output time:- Enter extrapolation limit in days (fractional if necessary): 5 Enter name for bore sample output file: model.smp - bore sample file model.smp written ok.

MF6MOD2OBS writes a site sample file named model.smp. As stated previously, this is the model-generated counterpart to observ.smp. (Actually head values in these two files are quite similar. This is because “observed heads” recorded in file observ.smp were generated using the parameter fields that we have been employing to date. This will not normally be the case, however.)

The above keyboard responses to MF6MOD2OBS’s prompts have been recorded in a file named mf6mod2obs.in. Hence the running of MF6MOD2OBS can be automated using the following command.

> mf6mod2obs < mf6mod2obs.in

### 5.3.3 Adding Observations to the PEST Control File

Tasks that await us are as follows:

- Creation of an instruction file to read the model output file model.smp.

- Addition of measurements contained in file observ.smp to the PEST control file case.pst which TPL2PST has just built for us.

- Creation of a batch file which PEST can run as the model.

Of course, there are often other tasks to do as well (for example making sure that weights in the PEST control file are correct). However the above tasks are the minimum required for our PEST input dataset to be acceptable to PEST.

The first two of the above tasks can be completed using the PESTPREP2 utility from the Groundwater Utility Suite. Run PESTPREP2, responding to its prompts as follows.

> pestprep2 Program PESTPREP2 adds observations to an existing PEST input dataset. These observation are based on a bore sample file prepared by MOD2OBS or SMP2SMP. Enter name of observation bore sample file: observ.smp Enter name of model bore sample file: model.smp Enter prefix for new observation names (8 chars or less): <Enter> Use numbers or bore identifiers for rest of observation names? [n/b]: b Use first 18 or last 18 characters of bore identifier? [f/l]: f Enter name for new observation group: heads Enter name for instruction file: model.ins - file model.ins written ok. Enter name of existing PEST control file: case.pst Enter name for new PEST control file: case1.pst - file case.pst read ok. - file case1.pst written ok.

Inspect file case1.pst to see what PESTPREP2 has just done. The PEST file looks complete! However there is one thing missing. Run PESTCHEK to tell you what it is.

> pestchek case1

The PEST control file needs a model command line. Before providing this command, we will create the model batch file.

## 5.4 The Model Batch File

The model that is run by PEST must have three components. One of these is the simulator itself (i.e. MODFLOW 6). However, prior to MODFLOW 6, PLPROC must be run in order for MODFLOW 6 input files to reflect current parameter values supplied by PEST. (There is only one MODFLOW 6 input file that is affected by these parameters in the present case; this is file kh.dat.) After MODFLOW 6 has been run, MF6MOD2OBS must be run to undertake temporal and spatial interpolation to measurement times and sites.

The entire model is encapsulated in file model.bat. This file has been prepared; it is reproduced below.

REM Delete intermediary files del kh.dat del gwmod.bhd REM Run the executables plproc plproc.dat mf6.exe mf6mod2obs < mf6mod2obs.in

Figure 5.1. The model batch file model.bat.

The first two commands in model.bat delete so-called “intermediate files”. These are files that are written by one program and read by another. If the pertinent program fails to run for some reason, then the following program will not read an old file, mistaking it for a new one.

Once these quality-assurance measures have been taken care of, model.bat runs PLPROC, MODFLOW 6 and MF6MOD2OBS in succession.

The PLPROC input file is plproc.dat. This contains a subset of the function calls that are made by plproc3.dat. (Unused function calls are commented out using the “##” string for easy recognition in this file.) There is no need to re-compute interpolation factors, nor write node data table files, when running PLPROC as part of a model that is being history-matched.

Add the command “model.bat” to the “model command line” section of file case1.pst. This section of the PEST control file should now be as follows.

* model command line model.bat

Figure 5.2. The “model command line” section of file case1.pst.

Now, if you use PESTCHEK to check the PEST input dataset again, it will give you the all-clear.

## 5.5 Running PEST

Now alter NOPTMAX from 50 to 0 in case1.pst. NOPTMAX is the first variable on the nineth line of this file. The “control data” section of case1.pst should now appear as below. (NOPTMAX is highlighted.)

pcf * control data restart estimation 227 39 3 0 1 2 1 single point 10.0 -3.0 0.3 0.03 10 lamforgive 10.0 10.0 0.001 0.1 0 0.005 4 4 0.005 4 0 0 0

Figure 5.3. The “control data” section of file case1.pst. NOPTMAX is highlighted.

Now run PEST as follows.

> pest case1

The objective function should be 19.94. (Model outputs no longer match observations because initial values for pilot point multiplier parameters in the PEST control file are now 1.0. This was requested by TPL2PST; see the TPL2PST input file tpl2pst.dat.)

If we wanted to calibrate our model, we would now need to add regularisation to the PEST control file. This is easily done using the ADDREG1 or ADDREG2 utility. Covariance matrices would then be required for the two sets of prior information equations (added by ADDREG1 or ADDREG2) that specify preferred values for the two sets of pilot point multiplier parameters.

Alternatively, case1.pst could be used by PESTPP-IES for Bayesian history-matching. However, we would first need to specify the collective prior probability distribution of our parameter set. This requires the same two covariance matrices.

Obviously, construction of these covariance matrices is our next task.

# 6. Prior Parameter Uncertainty

## 6.1 General

We will build a single covariance matrix. This will be used to specify prior uncertainties of both the Kh and Kz multiplier parameters. We will then assign a prior standard deviation to the only other parameter which features in the current inverse problem; this is the global Kz multiplier parameter. Ideally, the initial values of all of these parameters, as listed in the PEST control file, should be their prior preferred values. These, together with parameter covariance matrices and standard deviations, are sufficient to specify the prior parameter probability distribution. The workflow that is described in this tutorial ensures that this prior parameter probability distribution is an adequate manifestation of site concepts.

The covariance matrix that we are about to build must express an important feature of our study site. It must express the fact that correlation length, anisotropy and direction of anisotropy vary in space. We will assume that parameter variance is spatially uniform; however it would be no extra trouble to incorporate spatial variability of this hyperparameter into our workflow as well.

## 6.2 Interpolation from Conceptual Points to Pilot Points

### 6.2.1 Concepts

As was discussed at the start of this tutorial, hyperparameters which describe spatial variability of site hydraulic property stochasticity are assigned to 30 “conceptual points”; see Figure 3.2. However we need a covariance matrix for pilot point multiplier parameters; see Figure 2.5 for pilot point locations. So our first task is to interpolate hyperparameter values to these pilot points.

Using PLPROC, we have already interpolated conceptual hyperparameters to all active cells of the MODFLOW 6 grid. Recall that, when doing this, we went to some trouble to ensure that the interpolation process reflects the anisotropy that is embodied in the hyperparameters themselves. Hence the best way to obtain hyperparameter values at pilot points is to interpolate them from the model grid to these points. This is easily done, as every pilot point is situated close to at least one model cell centre.

There are a number of ways in which we could perform grid-to-pilot-point interpolation. For example, we could use MF6INTFAC, as was done above to interpolate from the model grid to observation locations. However we will maintain use of PLPROC, as we will shortly be using PLPROC to construct the covariance matrix.

### 6.2.2 The PLPROC Script

See file plproc4.dat. This is a continuation of plproc3.dat. Items comprising the augmented part of this script will now be described line by line. As usual, the description follows the script item.

hanis_value_ppm=new_plist(reference_clist=cl_ppm,value=1.0) hanis_bearing_ppm=new_plist(reference_clist=cl_ppm,value=0.0) a_ppm=new_plist(reference_clist=cl_ppm,value=1.0)

The above lines of script create new PLISTs. These all pertain to pilot points (embodied in the cl_ppm CLIST). These PLISTs must be created before values are interpolated to them from PLISTs belonging to other CLISTs.

hanis_value_ppm=hanis_ratio_mf6.ivd_interpolate_2d( & select=(idomain_mf6==1), & transform='none', & inv_power=2.0, & min_points=1, & max_points=4, & search_radius=100.0) hanis_bearing_ppm=hanis_bearing_mf6.ivd_interpolate_2d( & select=(idomain_mf6==1), & transform='none', & inv_power=2.0, & min_points=1, & max_points=4, & search_radius=100.0) a_ppm=a_mf6.ivd_interpolate_2d( & select=(idomain_mf6==1), & transform='none', & inv_power=2.0, & min_points=1, & max_points=4, & search_radius=100.0)

The above three calls to function ivd_interpolate_2d() perform inverse-power-of-distance interpolation from the MODFLOW 6 grid to each of the 113 pilot points. Interpolation is local because:

- the square of inverse distance is 2;

- onterpolation takes place from the closest 4 grid points to each pilot point.

Run PLPROC using the command.

> plproc plproc4.dat

## 6.3 Covariance Matrix Generation

plproc5.dat is a continuation of plproc4.dat.

covmat=cl_ppm.build_covmat_from_vario(variogram=exp, & a_plist=a_ppm, & sill=0.0625, & hanis_plist=hanis_value_ppm, & bearing_plist=hanis_bearing_ppm)

A covariance matrix is built using the build_covmat_from_vario() function. This function supports use of spatially varying hyperparameters. However, the sill is assigned a spatially uniform value of 0.0625. This is the square of 0.25 ( because variance is the square of standard deviation). Recall that the site conceptual model specifies this level of local Kh and Kz variability (applied to the logs of these quantities, and hence to the logs of parameter multipliers). The covariance matrix is given the name covmat.

covmat.write_matrix_to_file(file=covmat.dat)

Using this final PLPROC function call, the covariance matrix is recorded in a file named covmat.dat.

Run PLPROC as follows.

> plproc plproc5.dat

## 6.4 Parameter Uncertainty File

We need a parameter uncertainty file to complete our characterisation of prior parameter variability. This file has many uses. For example, it can be used by PESTPP-IES for sampling the prior parameter probability distribution. We will sample this distribution ourselves in a moment. Also, as is stated above, prior uncertainties (including covariance matrices that are used to characterise prior uncertainties) can be used for regularisation purposes if a MAP (maximum a posteriori) solution to the history-matching problem is sought through model calibration.

Specifications of a parameter uncertainty file are provided in Part B of the PEST manual. A parameter uncertainty file named param.unc has been prepared for you. It is reproduced below.

START STANDARD_DEVIATION kzmul_global 0.25 END STANDARD_DEVIATION START COVARIANCE_MATRIX file covmat.dat first_parameter khmul_pp1 last_parameter khmul_pp113 END COVARIANCE_MATRIX START COVARIANCE_MATRIX file covmat.dat first_parameter kzmul_pp1 last_parameter kzmul_pp113 END COVARIANCE_MATRIX

Figure 6.1. File param.unc.

The contents of this file are self-explanatory. Do not forget, however, that if a parameter is log-transformed in a PEST control file, then the uncertainty of that parameter which is expressed in a parameter uncertainty file must pertain to its log (to base 10). This applies to both parameter standard deviations and to parameter covariance matrices.

## 6.5 Generating Random Parameter Samples

### 6.5.1 Concepts

Now that we have a PEST control file and a parameter uncertainty file, we are can generate random parameter samples. The PLPROC script which serves as a MODFLOW 6 preprocessor can then be used to populate MODFLOW 6 Kh fields based on these random parameter samples. Because the PLPROC script embodies site concepts, these random Kh fields will comply with these concepts. They will therefore embody what we know about the site, at the same time as they embody that which we do not know about the site.

The PEST suite offers a number of options for random parameter set generation. Some of these options are included in PLPROC. However, we will use the RANDPAR utility. Some versions of this utility enable storage of random parameter sets in CSV and JCB files, ready for use by PESTPP-IES. However we will use RANDPAR to store 5 random parameter sets in 5 parameter value files. Then, with the help of our existing PLPROC script, we will obtain pictures of corresponding MODFLOW 6 Kh fields.

### 6.5.2 Running RANDPAR

Run RANDPAR, responding to its prompts as follows.

> randpar Enter name of existing PEST control file: case1.pst - 227 parameters read from file case1.pst. - 227 of these are adjustable. Use (log)normal or (log)uniform distrib for param generation? [n/u]: n Compute means as existing param values or range midpoints? [e/m]: e Respect parameter ranges? [y/n]: y Enter name of parameter uncertainty file: param.unc - covariance matrix file covmat.dat read ok. - covariance matrix file covmat.dat read ok. - parameter uncertainty file param.unc read ok. Enter name of parameter ordering file (<Enter> if none): <Enter> Enter filename base for parameter value files: random How many of these files do you wish to generate? 5 Enter integer random number seed (<Enter> if default): <Enter> - file random1.par written ok. - file random2.par written ok. - file random3.par written ok. - file random4.par written ok. - file random5.par written ok.

### 6.5.3 Running PEST

The easiest way to populate a MODFLOW 6 input file with a Kh field that is based on a random parameters set is to run PEST with NOPTMAX set to 0. But, before doing this, we must create a PEST control file in which initial parameter values are random parameter values. This is easily accomplished using the PARREP utility that is supplied with PEST.

However, before running PARREP, we need to alter the model slightly. Recall that model.bat employs an abbreviated script that does not record node data table files of Kh fields. We want to view these fields. So alter model.bat so that PLPROC runs the script contained in file plproc3.bat instead of that contained in plproc.bat. After you had made this alteration, model.bat should appear as below (with alterations highlighted).

REM Delete intermediary files del kh.dat del gwmod.bhd REM Run the executables plproc plproc3.dat mf6.exe mf6mod2obs < mf6mod2obs.in

Figure 6.1. Altered model.bat.

Run PARREP as follows to build a new PEST control file named case2.pst which is based on case1.pst.

> parrep random1.par case1.pst case2.pst

case2.pst now contains random parameter values from file random1.par as its initial parameter set. Now run PEST.

> pest case2

Our file of interest is the node data table file kh_kz.ndt. Copy this to kh_kz1.ndt.

> copy kh_kz.ndt kh_kz1.ndt

Now, if you like, repeat the above process four times to obtain kh_kz2.ndt, kh_kz3.ndt, kh_kz4.ndt and kh_kz5.ndt. Alternatively, the above procedure can be automated by placing it in a batch loop. See file random_loop.bat. It is reproduced below.

for /l %%i in (1,1,5) do ( parrep random%%i.par case1.pst case2.pst pest case2 copy kh_kz.ndt kh_kz%%i.ndt pause )

Figure 6.2. File random_loop.bat.

The “/l” following “for” on the first line of random_loop.bat instructs the operating system to execute the “for” loop in a numerical fashion (rather than on a set of files). When implementing these loops, the value of the %%i variable varies from 1 to 5 in steps of 1. The “pause” command requires that you press a key at the end of each loop traversal. This allows you to see what is going on.

Run the above batch file by typing its name at the screen prompt.

> random_loop.bat

On completion of the looped processing sequence, we have the five kh_kz*.ndt files that we want.

File kh_all.ndt gathers all of the Kh random fields into one large node data table file (together with cell IDOMAIN values). MF62GIS can now be used to build a MIF/MID file pair based on these Kh datasets. They can then be imported into a GIS or SURFER in ways that have already been described. The results are shown in Figure 6.3.

Figure 6.3. Five random realisations of MODFLOW 6 Kh fields.

Just to remind you how far we have come in this tutorial, Figure 6.4 shows the last of the above random Kh fields with conceptual points, pilot points and observation points superimposed.

Figure 6.4. Last of the Kh fields shown above with conceptual points, pilot points (red) and observation points (blue) superimposed.